AFNI Jazzercise

Please read the following questions, and use your AFNI know-how to answer them.  Hints to answering these questions are available in the “Hints” handout.  The answers to these questions can be found in the “Answers” handout.

Note these questions do not have to be done in order. Though these questions have a very rough order in terms of complexity and general usefulness, you can do whatever interests you and skip ahead to one that you like. Some exercises require longer computation time. Start the commands for those questions, and open another terminal window to continue on to the other questions. If you get stuck on a question, check the hints. If the hints don’t help, ask for help or go ahead and look at the answers (it’s not cheating; this is not a test).

- The dataset AFNI_data6/afni/func_slim+orig contains 7 sub-bricks of statistical data.  Use 3dbucket to create a smaller version of this dataset that contains only the sub-bricks:  #0, 3-6.  Name this new dataset some_stats.

Why: To understand the layout of AFNI datasets and sub-bricks. You’ll probably be interested only in specific sub-bricks and not all the sub-bricks that include baseline fit statistics.

- In directory AFNI_data6/afni you will find two anatomical datasets: anat+orig and second_anat+orig.  These datasets are two separate anatomical scans of a single subject.  They have already been aligned. Average them together into a single dataset called anat_mean+orig.  Notice that the result looks ‘cleaner’, since the noise has been reduced.

Why: averaging reduces the noise.

- Use two of AFNI’s programs that remove non-brain data, 3dAutomask and 3dSkullStrip, to remove data from outside of the brain from dataset AFNI_data6/afni/anat+orig.  Name the output file from 3dAutomask anat_auto+orig and the output file from 3dSkullStrip anat_3dSkull+orig. Compare the two output datasets. One of the results is much more subtle than the other, so compare each to the original and to each other with the Underlay and Overlay displays in the AFNI GUI. Note what “non-brain” data was removed by each. Did one program do a better job at skull stripping, or are the results similar? Note 3dSkullStrip may take a few minutes to run, so be patient. Run it in the background so you can get to the other questions while 3dSkullStrip is running. Also note 3dAutomask is not typically used for anatomical data, but it is particularly useful for EPI data.

Why: Removing the skull is useful for image registration and creating a brain-specific mask.

- Creating and Playing with ROI Masks:

  - The dataset AFNI_data6/afni/func_slim+orig has beta values and t-stats for 2 stimulus classes, Vrel and Arel.  Use 3dcalc to create a mask called VA_mask that is 1 everywhere that both the Vrel t-stat and the Arel t-stat values are greater than 4.2, and 0 everywhere else.

Why: Combining the results in a single mask is useful for a simple conjunction analysis.

  - Similar to part a, create a conjunction mask that is 1 wherever a>4.2 (from Vrel t-stat sub-brick), 2 wherever b>4.2 (from Arel t-stat sub-brick), 3 wherever both are true, and 0 otherwise.  Name this dataset VA_mask_4+orig (since it contains 4 values).

Why: Using masks with values of powers of two are easier to look at all possible combinations in conjunction analysis.

  - Use the afni GUI to display this mask, VA_mask_4+orig, so that each mask value gets its own color.  What does each color mean?

Why: The various combinations will each show a unique color that is easily visible and understood.

  - Use 3dROIstats to store the average time series from epi_r1+orig into the text file VA_mean.1D, where the mean is only over the voxels in the mask (from part a), VA_mask+orig.

Why: Mean time series curves for each ROI are useful for further analysis and display with other AFNI programs or with external applications like Excel and Matlab.

- Fun with 1D files:

  - Create three 1-column files with the numbers 1-10 in one column of the first file, 11-20 in the second file, and 21-30 in the third file.  (note: you might use 2 different AFNI programs to create each  of these files, but it can be done with only one)

  - Concatenate these 3 files into one 3-column file.  Call this 1D file 3_cols.1D.

  - Create a new file that contains columns 1, 2, 3, 3, 2,1, from part b  (i.e., there will be a total of 6 columns in this new 1D file).  Call this new 1D file 6_cols.1D.

  - Now take the 6 columns from the previous question and average them together to create a new file with a single column.  Call that new file ex_mean.1D.

Why: AFNI 1D programs can help to combine and analyze data from multiple voxels or masks.

- Fun with the AFNI GUI:

  - Open AFNI_data6/afni/anat+orig and in any one of the views (sagittal, axial, or coronal), change the gray-scale intensity range to be 300 minimum and 1200 maximum.

  - Open AFNI_data6/afni/func_slim+orig and set the Full-F as the OLay and Threshold.  Set the Threshold to F=8.0.  Show only Positive values and set the color scale to show only 8 colors.  Edit the color scale so that F-values between 45 and 90 are shown in lime green.

  - View the above settings you created from part b in a sagittal slice.  Make a jpeg file from sagittal slice #107 and name it cool_slide.

  - Switch to Talairach view and Talairach to the right cuneus.

  - Change the display to show 6 sagittal slices all at once, in a 3x2 montage.

  - Can you find the AFNI Mission statement hidden in the AFNI GUI?

Why: The AFNI GUI has the flexibility to look at your data, and it’s fun once you learn how.

- Doing Calculations in AFNI:

  - Determine what type of data (short, float, etc) makes up dataset AFNI_data6/afni/func_slim+orig.

Why: It’s important to know how to find information about your dataset.

  - Calculate 22.3 * 44.5 using the simple calculating program in AFNI.

Why: AFNI provides calculation tools including simple ones that are useful in scripts.

c.	Plot the function sin(x)/(1+x) for x=0 to 20 using command line AFNI programs.

    - Aligning data:

Align the anatomical dataset, AFNI_data6/anat+orig, with the EPI dataset, AFNI_data6/epi_r1+orig using the align_epi_anat.py script. Using the -AddEdge option for the script, examine the differences between before and after alignment in the AFNI GUI. Note this script can take a few minutes, so start it and continue with the other questions while the script is running.

Why: Alignment is important for knowing exactly where functional activation occurs. The quality of alignment is not always obvious and requires careful visual examination.

- Image Filtering:

- Smooth AFNI_data6/afni/epi_r1+orig with a 8mm FWHM filter.  Name the output file ex_blur8.

- Enhance AFNI_data6/afni/anat+orig by emphasizing the minimum-valued voxels across +/-3 voxels in the sagittal (z) direction.  Name the output dataset ex_minz3.  Note the effects on each slice direction.

- Enhance dataset ex_minz3+orig from part b by removing the noise with program 3danisosmooth.  Name the output dataset ex_aniso.  Use the -viewer option in this program to select the number of noise-removing iterations.

Why: Image filtering provides some powerful ways to enhance the data. Data can be changed in radically different ways. Use this power wisely and carefully.

- Random Exercises with AFNI Datasets:

- Open dataset AFNI_data6/afni/anat+orig dataset and find the spatial storage order (i.e., xyz-orientation).  Re-orient it to LPI orientation and name the new output dataset exLPI.

Why: Data may be required by specific programs to match other data or to match a specific orientation.

- Open dataset AFNI_data6/afni/func_slim+orig and create 2 separate datasets:  one with sub-brick 3 only and one with sub-brick 4 only. Call the former dataset ex_arel_coef and the latter ex_arel_tstat.

Why: Extracting specific data is useful for exporting to other software or for making the data fit in memory more easily if it’s a large dataset.

- Combine ex_arel_coef+orig and ex_arel_tstat+orig from part b into a single dataset called ex_fneg, having the fneg Coef for sub-brick 0, and fneg t-stat for sub-brick 1.

Why: Datasets can be manipulated to include only what you’re interested in.

- Convert dataset AFNI_data6/afni/func_slim+orig to Talairach coordinates with a 4mm3 resolution.  Use dataset anat+tlrc in the same directory as the data parent to perform the transformation on func_slim+orig.   Name the output file func_slim4mm.

Why: Talairach data will be 1mm3 by default. This resolution is often not necessary because it doesn’t reflect the resolution of the EPI data. It makes processing slower and taxes memory too.

- Locate the maximum “Full-F” stat voxel value in dataset func_slim4mm+tlrc and find the name of the Talairach atlas region that corresponds to that voxel’s position.

Why: Finding maximum activation can be scripted or searched interactively. Atlas regions should be used as a guide. The AFNI GUI includes various atlases that cite the regions associated with any specific voxel.

- Dataset AFNI_data6/afni/anat+orig was acquired sagittally and contains 124 slices.  Create a new dataset that contains only slices 40-90 of anat+orig.  Provide the new dataset with the prefix name anat_40_90.

Why: If memory or processing speed is a constraint, you can work with only part of the data.

- Volume Rendering:

  - Use the volume rendering plug-in to render the Talairached anatomical dataset.

  - Add an overlay of the func_slim dataset’s full F-stat (resampled to tlrc space) thresholded to a value of 10. Use the dataset created in 10d (or write out a new dataset from the GUI if you have skipped ahead).

  - Find an ROI that corresponds to the maximum F-stat location from the Talairach daemon atlas found in 10e (or eyeballed if you have skipped ahead).

Why: Volume rendering is good for getting a good idea of 3D locations and for making cool presentations.

- Simple statistics:

- The dataset, rall_func+orig under ~/AFNI_data6/afni/, is the output from the hands-on regression analysis, and it contains the t-statistic for the null hypothesis of "visual-reliable effect = auditory-reliable effect". Find out all the processing steps/scripts (including pre-processing and regression) that lead to this output, rall_func+orig.

- Find out in which sub-brick this t-statistic is stored in the output file. What is the range of these t- values and the degrees of freedom?

- What are the two-sided (or two-tailed) significance level (p-value) and FDR q-value corresponding to the t-statistic value of 4.25 with the degrees of freedom from question b? What is the one-sided (or one-tailed) significance level (p-value) for the alternative hypothesis of "visual-reliable effect < auditory-reliable effect"?

- What are the t-statistic and FDR q-value corresponding to a two-sided significance level (p-value) of 0.0001 with the degrees of freedom from question b? If only a one-sided alternative hypothesis (e.g., "visual-reliable effect < auditory-reliable effect") is considered but with the same significance level (p-value of 0.0001), what is the t-statistic and FDR q-value? Which one is more lenient or stringent, and which one should be adopted: one-sided or two-sided?

Why: Finding out where your statistics are and what they mean are vital for understanding and reporting your results.